# WEEK 7

## 🛠️ Step 1: Configure Tunnel Interface

### R1

interface tunnel 0
ip address 172.16.1.1 255.255.255.252
tunnel source g0/1
tunnel destination 200.1.1.6

### R2

interface tunnel 0
ip address 172.16.1.2 255.255.255.252
tunnel source g0/1
tunnel destination 200.1.1.2

## 🛠️ Step 2: Route Traffic Through Tunnel

ip route 192.168.2.0 255.255.255.0 172.16.1.2

## ✅ Verification

show interface tunnel 0

ping remote LAN

✔ Offices communicate as if locally connected.