WEEK 5: Multi-Area OSPF Configuration(10)

🎯 Objective

- Implement scalable routing using OSPF Multi-Area Design

- Reduce routing overhead

- Configure backbone (Area 0) and non-backbone areas

🗺️ Topology (Example)

LAN1 --- R1 --- R2 --- R3 --- LAN2 |Area 0|Area 1|

🛠️ Step 1: Configure IP Addressing

R1

interface g0/0 ip address 192.168.1.1 255.255.255.0 no shutdown interface g0/1 ip address 10.0.12.1 255.255.255.252 no shutdown

R2

interface g0/0 ip address 10.0.12.2 255.255.255.252 no shutdown

interface g0/1 ip address 10.0.23.1 255.255.255.252 no shutdown

R3

interface g0/0 ip address 10.0.23.2 255.255.255.252 no shutdown interface g0/1 ip address 192.168.2.1 255.255.255.0 no shutdown

🛠️ Step 2: Configure OSPF Areas (10 Marks)

R1 (Backbone Area 0)

router ospf 1 network 192.168.1.0 0.0.0.255 area 0 network 10.0.12.0 0.0.0.3 area 0

R2 (ABR – connects two areas)

router ospf 1 network 10.0.12.0 0.0.0.3 area 0 network 10.0.23.0 0.0.0.3 area 1

R3 (Area 1)

router ospf 1 network 10.0.23.0 0.0.0.3 area 1

network 192.168.2.0 0.0.0.255 area 1

✅ Verification

show ip ospf neighbor show ip route ospf